# The State of Mass Spectrometry-Centered Bottom-Up Proteomics: A 2026 Analytical Deep Dive and Computational Framework

The analytical landscape of bottom-up proteomics as of early 2026 represents a paradigm of high-resolution molecular characterization, where the identification and quantification of the proteome are achieved through the systematic fragmentation and measurement of peptides. This field has transitioned from a specialized discovery tool to a cornerstone of precision medicine, clinical diagnostics, and systemic biological inquiry. The current state of the art is defined by a convergence of advanced liquid chromatography (LC), high-sensitivity mass spectrometry (MS) hardware, and a computational revolution driven by decentralized machine learning (ML) models and standardized data formats. Bottom-up proteomics, by its very nature, relies on the piecewise reconstruction of proteins from their constituent peptides, a process that necessitates extreme precision in both the wet-lab preparation and the dry-lab informatics.1

## The Biochemical Foundation: Innovations in Sample Preparation and Proteolysis

The efficacy of any proteomic experiment is irrevocably tethered to the quality of the initial sample preparation. In 2026, the industry has moved beyond rudimentary protein extraction toward sophisticated, automated workflows that minimize loss and maximize reproducibility. Sample preparation constitutes a significant portion of the analytical workflow and demands substantial manual effort, though the increasing ubiquity of robotic handling systems like the Biomek i7, Opentrons, and Hamilton workstations has begun to mitigate human error and enhance throughput.2

## The Evolution of Proteolytic Digestion

Trypsin remains the predominant protease in bottom-up workflows due to its high specificity and the generation of peptides with favorable $C$-terminal basic residues that facilitate efficient ionization and fragmentation.1 However, the drive for deeper proteome coverage has led to the adoption of multi-enzyme strategies. Complementary proteases such as Lys-C, Glu-C, Asp-N, and Arg-C are now frequently employed to uncover protein regions that are inaccessible to trypsin alone, such as extremely hydrophobic domains or regions lacking lysine and arginine residues.1 These alternative enzymes are particularly critical for mapping post-translational modifications (PTMs) and identifying protein isoforms that might otherwise remain "invisible" in a standard tryptic digest.1

| Protease | Cleavage Specificity | Primary Application in 2026 |
| --- | --- | --- |
| Trypsin | $C$-term Lys, Arg | Standard discovery and quantification |
| Lys-C | $C$-term Lys | Pre-digestion or limited proteolysis |
| Glu-C | $C$-term Glu, Asp | Mapping of acidic domains |
| Asp-N | $N$-term Asp, Glu | Alternative fragmentation for PTM mapping |
| ProAlanase | Proline-specific | Analysis of proline-rich regions |
| Chymotrypsin | Phe, Tyr, Trp | Hydrophobic protein analysis |

Current innovations include the use of recombinant trypsin, which exhibits improved specificity and resistance to autoproteolysis, and the development of fast digestion kits that reduce processing times from overnight to as little as four hours without sacrificing efficiency.1 This reduction in time is a critical bottleneck for clinical research, where rapid turnaround is required for biomarker verification.4

## Advanced Sample Enrichment and Low-Input Workflows

Breakthroughs in handling complex biological matrices have been largely achieved through technologies like suspension-trapping (S-Trap) and solvent-induced protein capture on magnetic beads (SP3/PAC).1 These methods allow proteins to be solubilized in harsh detergents or denaturants—essential for membrane proteins—which are then washed away prior to digestion. The compatibility of magnetic bead-based technologies with automated liquid handling systems has made them especially promising for high-throughput clinical applications.1

A profound shift has occurred in single-cell proteomics, where the challenge is to analyze the roughly 200 picograms of protein contained within a single mammalian cell.1 Workflows such as nPOP, nanoPOTS, and Chip-Tip have matured, utilizing nanoliter-volume processing to minimize surface adsorption and sample loss.1 When paired with ultra-sensitive mass spectrometers, these automated systems enable the identification of thousands of proteins from individual cells, allowing researchers to resolve the dramatic differences in protein expression and modification states that characterize rare or transient cellular behaviors.1

## Analytical Separations and the Integration of Ion Mobility

The resolution of complex peptide mixtures relies on the synergy between liquid chromatography and the mass spectrometer. Modern liquid chromatography systems are increasingly optimized for speed, utilizing shorter gradients (5–15 minutes) to meet the demands of high-throughput clinical cohorts.5

## High-Resolution Ion Mobility (HRIM) and SLIM

A major advancement in 2025–2026 is the integration of High-Resolution Ion Mobility (HRIM), specifically using Structures for Lossless Ion Manipulation (SLIM) technology.7 Traditional mass spectrometry often loses up to 99% of ions during precursor isolation in the quadrupole. HRIM-based precursor isolation, particularly in Parallel Accumulation-Mobility Aligned Fragmentation (PAMAF) mode, offers a way to avoid these losses while increasing the rate at which precursor ions are fragmented and detected.7 PAMAF mode utilizes mobility-based time alignment to associate fragment ions with their corresponding precursors, significantly reducing the frequency of chimeric spectra and enabling the detection of over 8,000 protein groups in a single analysis.7

## The Impact of Shorter LC Gradients

While shorter gradients enhance throughput, they also reduce chromatographic peak widths, requiring faster mass analyzers to maintain quantitative accuracy.5 At least six data points per chromatographic peak are typically necessary to accurately define the peak shape and estimate the peak apex for robust quantification.5 Failure to achieve this sampling rate leads to increased missing values and poor reproducibility across replicates. Consequently, the development of faster Orbitrap and Time-of-Flight (TOF) instruments, alongside trapped ion mobility spectrometry (TIMS) and high-field asymmetric waveform ion mobility spectrometry (FAIMS), has been essential for the success of high-throughput workflows.5

## Mass Spectrometry Acquisition Strategies: The DDA and DIA Paradigm

The choice of data acquisition mode remains a pivotal decision in bottom-up proteomics. Data-Dependent Acquisition (DDA) has historically been favored for its low spectral complexity, where the instrument selects the $N$ most intense precursor ions for fragmentation. However, DDA is limited by its semi-stochastic nature, which often results in inconsistent identification across samples.8

## The Rise of Data-Independent Acquisition (DIA)

Data-Independent Acquisition (DIA) has emerged as the cornerstone method for comprehensive and reproducible proteomic analysis.5 Unlike DDA, DIA systematically fragments all ions within predefined $m/z$ windows, ensuring that a digital record of the entire sample is captured.9 This consistency yields fewer missing values across cohort studies, enhancing statistical power for longitudinal and clinical research.8

| Acquisition Feature | Data-Dependent Acquisition (DDA) | Data-Independent Acquisition (DIA) |
| --- | --- | --- |
| Precursor Selection | Intensity-based (stochastic) | Systematic $m/z$ windows |
| Quantitative Data | Fragment ion and MS1 | Comprehensive fragment ion |
| Missing Values | Higher across replicates | Minimal due to systematic scanning |
| Spectral Complexity | Lower (clean MS2) | Higher (chimeric MS2) |
| Library Requirement | Standard database search | Spectral or predicted libraries |

Recent innovations in DIA, such as diaPASEF and scanning quadrupole-based implementations (e.g., ZenoTOF ZT Scan), have pushed the boundaries of sensitivity and depth.5 These methods are particularly effective when combined with in silico-predicted spectral libraries, which promote projects where limited sample amount or measurement time makes the generation of experimental libraries unfeasible.8

## Computational Proteomics and the Machine Learning Revolution

The "scaling law" of machine learning has found fertile ground in proteomics, where large volumes of accurate mass spectrometry data have enabled the development of high-performance deep learning models.2 These models can predict peptide properties—such as fragmentation patterns, retention times, and collisional cross-sections—with remarkable accuracy, fundamentally changing how mass spectra are interpreted.12

## Decentralized Prediction Services: Koina

A landmark development in the computational space is the introduction of Koina, an open-source, decentralized, and web-accessible high-performance prediction service.12 Koina democratizes access to state-of-the-art models like Prosit, MS2PIP, DeepLC, and AlphaPeptDeep by allowing researchers to fetch predictions via simple HTTP/S requests.12 This design removes the need for local GPU-intensive hardware, enabling even small laboratories to integrate advanced ML/DL into their pipelines.12

Koina supports standardized input formats like the PSI ProForma 2.0 notation, which provides a human-readable and machine-parsable way to represent peptidoforms with complex modifications.13 This interoperability is essential for the growing field of PTM proteomics, where site localization is a major analytical challenge.15

## Advanced Rescoring and Peptide Identification

Spectrum interpretation remains prone to ambiguities, especially in complex search spaces where peptides exhibit similar fragmentation patterns and close retention times.17 This is particularly true for positional isomers of modified peptides—"peptide twins" that share the same mass but differ in the site of phosphorylation or glycosylation.17

Tools like MS2Rescore address these ambiguities by integrating search engine results with machine learning-derived features.18 By comparing observed spectral intensities and retention times with predictions from models like DeepLC and MS2PIP, MS2Rescore significantly boosts identification sensitivity and specificity.19 These advancements allow for more confident peptide assignments, even in metaproteomics or immunopeptidomics where the search space is vast and often contains homologous sequences.17

## Data Standards and the Quest for Quality Control

As proteomics data scales to tens of thousands of datasets in repositories like PRIDE, the need for robust, standardized quality control (QC) and metadata management has become paramount.20

## The mzQC Standard and Tooling

The Proteomics Standards Initiative (PSI) has developed mzQC, a standardized JSON format for reporting and archiving quality control metrics.22 mzQC captures a wide range of parameters, including MS1/MS2 scan counts, TIC duration, and mass error distributions, defined through a controlled vocabulary (QC CV) to ensure semantic consistency.24

| mzQC Metric Type | Example Metric | Significance |
| --- | --- | --- |
| Acquisition | MS1/MS2 scan counts | Monitors duty cycle and instrument speed |
| Performance | Mass error (ppm) | Checks instrument calibration and stability |
| Chromatographic | TIC duration/RT-Duration | Evaluates LC gradient and separation |
| Quantitative | Identification rate | Assesses sample preparation and complexity |

Libraries like pymzqc (Python) and rmzqc (R) facilitate the creation and use of these files, allowing for the integration of QC into automated analysis workflows.23 Newer tools like pmultiqc leverage the widely adopted MultiQC framework to generate interactive, web-based reports that are metadata-aware, using the Sample Data Relationship Format (SDRF) to guide metrics according to experimental design.21

## Public Data Repositories and Data Reuse

The ProteomeXchange consortium serves as the global infrastructure for proteomic data sharing, with member databases like PRIDE (EBI), MassIVE (UCSD), and PeptideAtlas providing access to over 64,000 datasets.20 The recent introduction of pridepy, a Python client for the PRIDE Archive, enables researchers to programmatically search, retrieve, and download large-scale datasets via protocols like Aspera, Globus, and FTP.28 This programmatic access is vital for "repository-scale" analysis, where public data is reanalyzed to train new machine learning models or to validate clinical biomarkers.28

## Challenges and Technical Bottlenecks in 2026

Despite the maturity of the field, several challenges persist that define the current research frontiers.

## The "Invisible" Proteome and Ambiguity

The "invisible" proteome refers to protein regions or entire proteins that are missed due to digestion bias, low abundance, or spectral ambiguity.1 The prevalence of indistinguishable peptides in the human canonical proteome remains high; many peptides cannot be uniquely assigned because they share identical mass and near-identical fragment patterns with other sequences.17 The MSCI (Mass Spectrometry Content Information) library was developed specifically to assess the information content of spectra and identify these "peptide twins," helping researchers prioritize more informative fragment ions for quantification.17

## Site Localization of PTMs

Determining the precise site of a post-translational modification remains complex, especially when multiple acceptor residues are present.15 While algorithms like Ascore and tools such as pyAscore provide probabilistic scoring based on site-determining ions, the field is still moving toward a more robust, "modification-aware" identification framework.15 Modern software like Spectronaut integrates confirming and refuting fragment-ion signals into localization probability scores, which are essential for understanding the functional effects of PTMs on protein activity.16

## The Computational Burden of Infrastructure

The growing volume of raw MS data imposes significant logistical and financial pressure on laboratory infrastructure. Scalable computational frameworks are needed not just for initial processing, but also for storage, retrieval, and integration across multi-omics projects.30 The adoption of columnar, Parquet-based designs for data storage has improved compression and query performance, facilitating interoperability with contemporary data science tools.31

## Insights into Future Outlooks and Clinical Integration

The integration of proteomics into clinical diagnostics is contingent upon the development of standardized, reproducible workflows that can handle large patient cohorts. The shift toward shorter gradients and automated sample preparation is a direct response to this need.4 Furthermore, the emergence of "controlled quantitative experiments" (CQEs) for parameter optimization allows researchers to evaluate quantitative precision and accuracy objectively, moving beyond simple identification counts as a metric of success.6

As machine learning models continue to refine our ability to predict the physical properties of peptides, the reliance on experimental spectral libraries is decreasing. This shift toward in silico-predicted libraries allows for the re-interrogation of existing data with new biological hypotheses, a process known as retrospective mining.16 The synergy between high-sensitivity instrumentation (like the Orbitrap Astral or timsTOF Ultra) and these intelligent computational frameworks is rapidly closing the gap in our ability to fully map the human proteome in a high-throughput, quantitative manner.

## To address the challenges of data quality verification and the visualization of peptide-level identifications in a structural context, the following project is proposed for implementation.Coding Project Proposal: ProteoStruct-QC

## Project Title: ProteoStruct-QC - An Interactive Structural Mapping and QC Dashboard

The Challenge:

Researchers often struggle to reconcile mass spectrometry quality metrics (like identification rates and mass error) with the biological and structural significance of their data. While tools exist for QC (like mzQC) and structural visualization (like Mol*), they are often fragmented. There is no simple, "all-in-one" Python-based tool that allows a user to download a public dataset, check its quality, and immediately see where the identified peptides map onto the 3D structure of the parent protein.

The Solution:

ProteoStruct-QC is a Streamlit-based web application that integrates PRIDE data access, standardized QC parsing, and 3D molecular visualization.

## Implementation Blueprint

- Environment and Tools:

  - IDE: VS Code with Claude Code (for rapid prototyping and code generation).

  - Language: Python 3.10+.

  - Libraries:

    - pridepy: For programmatic retrieval of project data from PRIDE.28

    - pymzqc: For parsing and displaying standardized quality metrics.26

    - pyteomics: For peptide sequence handling and theoretical mass calculations.14

    - streamlit-molstar: For 3D visualization of protein structures and peptide mapping.34

    - pandas & plotly: For data manipulation and interactive QC charting.35

- Implementation Logic:

  - Data Intake: The user provides a PRIDE accession (e.g., PXD070049). The app uses pridepy to fetch metadata and identifying whether an mzQC file is available.

  - QC Engine: If mzQC is found, pymzqc extracts metrics like "Number of MS1/MS2 scans" and "Mass Error Histogram." These are plotted using Plotly.

  - Structural Connector: The user selects an identified protein. The app uses the UniProt ID to fetch the corresponding PDB or AlphaFold structure.

  - Peptide Mapping: Identified peptides are highlighted on the 3D structure using streamlit-molstar. Peptides with PTMs are colored distinctly to show site localization.37

  - Ambiguity Check: The app utilizes MSCI-like logic to highlight peptides that are "ambiguous" (i.e., map to multiple proteins), alerting the user to potential quantification errors.17

- Usefulness and Fun:

  - For the Researcher: It provides a one-click validation of public data before they invest time in reanalysis.

  - For the Student: It makes the "invisible" world of mass spectrometry data tangible by connecting abstract peptide strings to physical 3D shapes.

  - Fun Factor: Interactive 3D structural mapping and the ability to "scout" public datasets effortlessly.

## Implementation Guide for the IDE/Coding Agent

- Prompt for Claude Code: "Create a Streamlit app that uses pridepy to fetch project files for a given PXD accession. Integrate pymzqc to parse any .mzQC files found and display the 'Number of MS2 spectra' in a Plotly bar chart. Add a sidebar where the user can input a UniProt ID, and use streamlit-molstar to render the structure, highlighting a list of peptide sequences passed in a text area."

- Data Source: All data is fetched from the PRIDE Archive API.38

- Prerequisites: Low. Requires basic Python knowledge and an internet connection to reach the EBI servers.

## Detailed Technical Context for Implementation

To ensure the proposed ProteoStruct-QC project meets professional standards, the following technical details regarding the libraries and data formats should be considered.

## Handling mzQC in Python

The pymzqc library provides an object-oriented representation of the JSON schema.26 A typical implementation to extract a metric would involve:

Python

from mzqc import MZQCFile as qc

def get_ms2_count(mzqc_path):
    with open(mzqc_path, 'r') as f:
        data = qc.from_json(f.read())
    # Iterate through runQualities to find the MS2 count metric
    for metric in data.runQualities.qualityMetrics:
        if metric.accession == "QC:4000060": # Accession for MS2 count
            return metric.value
    return None

This logic ensures that the application adheres to the PSI standards for quality reporting.25

## Structural Visualization with Streamlit-Molstar

The integration of streamlit-molstar is straightforward but powerful.34 It allows the developer to pass remote URLs (like those from the PDBe or AlphaFold DB) directly to the viewer. For mapping peptides, the "Sequence Matching" feature can be leveraged to automatically highlight regions of the structure that correspond to the identified sequences.41

$\text{Peptide Highlight Color Logic}:$

- Green: High identification confidence (Percolator $q$-value $< 0.01$).42

- Yellow: Moderate confidence.

- Red: Ambiguous peptide (maps to multiple proteins).17

- Magenta: Peptide with identified PTM (e.g., phosphorylation).15

## The Role of Public Benchmark Datasets

The project can be pre-loaded with successful benchmarks like PXD070049, which contains high-throughput DIA data from a triple-proteome mixture (Human/Yeast/E. coli).6 This allows the user to explore a "gold standard" dataset and see how the QC metrics look in a high-quality run versus a lower-quality public submission.

## Synthesis of Computational Insights

The ProteoStruct-QC project is not merely a visualization tool; it is a gateway to the broader infrastructure of modern proteomics. By requiring the project lead to rely on public data and open standards, the project reinforces the principles of open science and data reuse that are driving the field forward.20 The integration of decentralized machine learning via Koina could be a "Version 2.0" feature, where predicted intensities are overlaid on the experimental spectra for visual validation of identifications.12

In conclusion, the field of bottom-up proteomics in 2026 is a highly integrated discipline where the boundaries between biochemistry, analytical chemistry, and computer science have blurred. The success of an experiment now depends as much on the choice of the correct rescoring algorithm (like MS2Rescore) as it does on the choice of the correct protease.1 The proposed ProteoStruct-QC project provides a practical and engaging way to navigate this complexity, offering a tool that is both academically rigorous and accessible to the common user.

#### Works cited

- Advances in Bottom-Up Proteomics | Biocompare: The Buyer's Guide for Life Scientists, accessed on March 19, 2026, https://www.biocompare.com/Editorial-Articles/623028-Advances-in-Bottom-Up-Proteomics/

- Recent Advances in Mass Spectrometry-Based Bottom-Up Proteomics | Analytical Chemistry, accessed on March 19, 2026, https://pubs.acs.org/doi/10.1021/acs.analchem.4c06750

- Recent Advances in Mass Spectrometry-Based Bottom-Up ..., accessed on March 19, 2026, https://pmc.ncbi.nlm.nih.gov/articles/PMC12117541/

- Full article: Optimization of bottom-up proteomic sample preparation by design of experiments for absolute quantification of 257 proteins in human plasma using UPLC-MRM-MS - Taylor & Francis, accessed on March 19, 2026, https://www.tandfonline.com/doi/full/10.1080/14789450.2025.2504994%4010.1080/tfocoll.2026.0.issue-IERU_Clinical-Translational-Omics-Symposium-2023

- Recommendations for Quantitative Data-Independent Acquisition ..., accessed on March 19, 2026, https://www.biorxiv.org/content/10.1101/2025.09.22.677725v1.full-text

- (PDF) LFQ Benchmark Dataset - Generation Beta: Assessing Modern Proteomics Instruments and Acquisition Workflows with High-Throughput LC Gradients - ResearchGate, accessed on March 19, 2026, https://www.researchgate.net/publication/400393284_LFQ_Benchmark_Dataset_-_Generation_Beta_Assessing_Modern_Proteomics_Instruments_and_Acquisition_Workflows_with_High-Throughput_LC_Gradients

- Recent Advances in Mass Spectrometry-Based Bottom-Up Proteomics - ResearchGate, accessed on March 19, 2026, https://www.researchgate.net/publication/389324443_Recent_Advances_in_Mass_Spectrometry-Based_Bottom-Up_Proteomics

- Data-Independent Acquisition: A Milestone and Prospect in Clinical Mass Spectrometry–Based Proteomics - PMC, accessed on March 19, 2026, https://pmc.ncbi.nlm.nih.gov/articles/PMC11380018/

- Recent implementations of data-independent acquisition for cancer biomarker discovery in biological fluids - Taylor & Francis, accessed on March 19, 2026, https://www.tandfonline.com/doi/full/10.1080/14789450.2025.2491355?utm_source=researchgate.net&utm_medium=article

- Optimizing MS Parameters for Data-Independent Acquisition (DIA) to Enhance Untargeted Metabolomics | Journal of Proteome Research - ACS Publications, accessed on March 19, 2026, https://pubs.acs.org/doi/10.1021/acs.jproteome.5c00622

- Evaluation of DDA Library-Free Strategies for Phosphoproteomics and Ubiquitinomics Data-Independent Acquisition Data | Journal of Proteome Research - ACS Publications, accessed on March 19, 2026, https://pubs.acs.org/doi/10.1021/acs.jproteome.2c00735

- Koina: Democratizing machine learning for proteomics research - PMC - NIH, accessed on March 19, 2026, https://pmc.ncbi.nlm.nih.gov/articles/PMC11185529/

- (PDF) Koina: Democratizing machine learning for proteomics research - ResearchGate, accessed on March 19, 2026, https://www.researchgate.net/publication/397512916_Koina_Democratizing_machine_learning_for_proteomics_research

- Proteomics Standards Initiative's ProForma 2.0: Unifying the Encoding of Proteoforms and Peptidoforms - ACS Publications, accessed on March 19, 2026, https://pubs.acs.org/doi/10.1021/acs.jproteome.1c00771

- A Python Package for the Localization of Protein Modifications in Mass Spectrometry Data, accessed on March 19, 2026, https://pmc.ncbi.nlm.nih.gov/articles/PMC9898206/

- PTM Proteomics Analysis Workflow: Variable Modifications, Site Localization, and FDR Control - MetwareBio, accessed on March 19, 2026, https://www.metwarebio.com/ptm-proteomics-analysis-workflow-site-localization-fdr-control/

- MSCI: an open-source Python package for information content assessment of peptide fragmentation spectra - Repositori UPF, accessed on March 19, 2026, https://repositori.upf.edu/bitstreams/ac1bbbda-89b9-428c-bbc8-032cd644476b/download

- MS²Rescore 3.0 is a modular, flexible, and user-friendly platform to boost peptide identifications, as showcased with MS Amanda - ChemRxiv, accessed on March 19, 2026, https://chemrxiv.org/doi/pdf/10.26434/chemrxiv-2023-rvr9n-v2

- Boosting metaproteomics identification rates and taxonomic specificity with MS2Rescore, accessed on March 19, 2026, https://www.biorxiv.org/content/10.1101/2025.02.17.638783.full

- The ProteomeXchange consortium in 2026: making proteomics data FAIR - PubMed - NIH, accessed on March 19, 2026, https://pubmed.ncbi.nlm.nih.gov/41206473/

- pmultiqc: An open-source, lightweight, and metadata-oriented QC reporting library for MS proteomics - bioRxiv.org, accessed on March 19, 2026, https://www.biorxiv.org/content/10.1101/2025.11.02.685980v1.full.pdf

- mzQC - HUPO Proteomics Standards Initiative, accessed on March 19, 2026, https://www.psidev.info/mzqc

- rmzqc: Creation, Reading and Validation of 'mzqc' Files - ms-quality-hub, accessed on March 19, 2026, https://ms-quality-hub.r-universe.dev/rmzqc

- Introduction to mzQC - HUPO-PSI GitHub Pages, accessed on March 19, 2026, https://hupo-psi.github.io/mzQC/mzQC-intro/

- mzQC format for analytical chemists - HUPO-PSI GitHub Pages, accessed on March 19, 2026, https://hupo-psi.github.io/mzQC/use-cases/analytical-chemists/

- MS-Quality-Hub/pymzqc: The python implementation to mzqc - GitHub, accessed on March 19, 2026, https://github.com/MS-Quality-Hub/pymzqc

- The ProteomeXchange consortium in 2026: making proteomics data FAIR - PMC, accessed on March 19, 2026, https://pmc.ncbi.nlm.nih.gov/articles/PMC12807779/

- pridepy: A Python package to download and search data from PRIDE database - Open Journals, accessed on March 19, 2026, https://www.theoj.org/joss-papers/joss.07563/10.21105.joss.07563.pdf

- (PDF) pridepy: A Python package to download and search data from PRIDE database, accessed on March 19, 2026, https://www.researchgate.net/publication/390134494_pridepy_A_Python_package_to_download_and_search_data_from_PRIDE_database

- Perspectives in Computational Mass Spectrometry: Recent Developments and Key Challenges - ChemRxiv, accessed on March 19, 2026, https://chemrxiv.org/doi/pdf/10.26434/chemrxiv-2025-h6q1k

- Perspectives in computational mass spectrometry: recent developments and key challenges | Bioinformatics Advances | Oxford Academic, accessed on March 19, 2026, https://academic.oup.com/bioinformaticsadvances/article/5/1/vbaf301/8382609

- Recommendations for Quantitative Data-Independent Acquisition (DIA) Proteomics using Controlled Quantitative Experiments (CQEs) - bioRxiv.org, accessed on March 19, 2026, https://www.biorxiv.org/content/10.1101/2025.09.22.677725v2

- Roestlab/PythonProteomics: A collection of available Python tools for Proteomics analysis, accessed on March 19, 2026, https://github.com/Roestlab/PythonProteomics

- New component: Streamlit-Molstar for visualisation and analysis of large-scale molecular data, accessed on March 19, 2026, https://discuss.streamlit.io/t/new-component-streamlit-molstar-for-visualisation-and-analysis-of-large-scale-molecular-data/67631

- Data Visualization using Streamlit, accessed on March 19, 2026, https://towardsdatascience.com/data-visualization-using-streamlit-151f4c85c79a/

- Building a dashboard in Python using Streamlit - Show the Community!, accessed on March 19, 2026, https://discuss.streamlit.io/t/building-a-dashboard-in-python-using-streamlit/60621

- streamlit-molstar - GitHub, accessed on March 19, 2026, https://github.com/pragmatic-streamlit/streamlit-molstar

- PRIDE Database API Documentation - EMBL-EBI, accessed on March 19, 2026, https://www.ebi.ac.uk/pride/markdownpage/prideapi

- PRIDE Archive Data Downloading - EMBL-EBI, accessed on March 19, 2026, https://www.ebi.ac.uk/pride/markdownpage/pridefiledownload

- single-run.mzQC.md - GitHub Gist, accessed on March 19, 2026, https://gist.github.com/mwalzer/351ef729ce3fc57e6246e43a30f09e62

- Nano Protein Viewer VSCode Extension - Visual Studio Marketplace, accessed on March 19, 2026, https://marketplace.visualstudio.com/items?itemName=StevenYu.nano-protein-viewer

- ms2rescore.report - About MS²Rescore - Read the Docs, accessed on March 19, 2026, https://ms2rescore.readthedocs.io/en/stable/api/ms2rescore.report/

- Koina, accessed on March 19, 2026, https://koina.wilhelmlab.org/docs/